HF-AUTO firmware update procedure:

*1 Set up a file folder

* right click on the file ICON at the bottom of the screen

* left click on "File Explorer"

* left click on This PC

* double click on OS (C:) drive

* in this window point to a location outside any file and right click

* on the drop down list move pointer over New then move pointer over Folder and left click

* enter the new file name "HF_AUTO" and left click on an open location to end this operation

*2 Down load the latest HF-AUTO firmware

* left click on Microsoft Edge (Internet Explorer)

* enter web site name "www.Palstar.com" then press "Enter"

* point to "AUTO TUNERS" and left click

* point to "HF-AUTO-R" and left click

* point to "HF-AUTO FIRMWARE x.xx (TUNER) and left click

* point to "View downloads" in the box that appears near the bottom of this window and left click

* point to "HF_AUTO_x_xx_Load.zip" and left click

* in the window that opens up point to "HF_AUTO_x_xx_Load" and right click

* in the drop down list that appears left click "Copy"

* select the the "HF_AUTO" folder that was previous create above and double click

* pointing inside this folder right click

*  in the drop down list that appears left click "Paste"

*3 set up the USB to Serial RS232 adapter

* right click on the file ICON at the bottom of the screen

* left click on "File Explorer"

* point to "This PC" and right click

* in the drop down list point to "Properties" and left click

* left click on "Device Manager"

* plug in the USB adapter into the computer

* a "USB Serial Port" will appear

* wait until the computer automatically finds the proper driver for this device

* This may take some time.

* The yellow triangle will go away and a COM number will be assigned by the computer.

* The COM number assigned will appear after the "USB Serial Port".

*4 match the COM number with the HF-AUTO load software by doing 4A or 4B

*4A change the assigned COM number to COM1 (matching the default in the HF-AUTO load software)

* in the "Device Manager" window double click "USB Serial Port"

* left click on "Port Settings"

* Left click on "Advanced"

* select the drop down menu next to "COM Port Number:"

*  select COM1 then left click on "OK"

*4B change the COM number used in the HF-AUTO load software to match the COM number being used

* select the file folder contents that was done in step 2

* point to the file "HF_script" and right click

* in the drop down list point to "Open With" then point to "Notepad" and left click

* change the "COM1" to "COMx" where x = the COM number that was found in step 3

* left click on 'File" then left click on "Save"

* close the Notepad Window

*5 Program the HF-AUTO with new firmware.

* apply power to the HF-AUTO

* change the slide switch in the back of the HF-AUTO to PROGRAM (red Program LED should be on)

* connect the DB9 adapter cable end to the HF-AUTO

* select the file folder contents that was done in step 2

* double click on "load_HF_9600" file

* a DOS Window will open and the HF-AUTO will be in the process of updating the firmware.

* this DOS Window will remain open for over a minute and will automatically close when complete.

* unplug the adapter from the HF-AUTO and return the slide switch to "NORMAL"